# OBD Modes Requirements Document

Contents

Document Control	3

Document History	4

Introduction	5

Functional Description	5

Requirements	5

References	10

# Document Control

- Document Title: Transport Protocol for CAN

- Version: 1.0

- Date: 1/4/25

- Author: Akshay Naik

- Reviewed by: [Enter Reviewer Name]

- Approved by: [Enter Approver Name]

# Document History

| Author | Date | Revision | Changes |
| --- | --- | --- | --- |

# Introduction

The CAN Transport Protocol (CAN-TP), standardized under ISO 15765-2, is a communication protocol used in automotive diagnostics to enable the transmission of large data packets over the Controller Area Network (CAN) bus. Since the CAN protocol inherently supports a maximum payload of only 8 bytes per frame, CAN-TP extends this capability by segmenting larger messages into multiple CAN frames and reassembling them at the receiving end.

# Functional Description

This document details the functional behaviour of the transport protocol as specified in ISO 15765-2. It includes requirements for:

- Data transmission and reception over CAN.

- Segmentation and reassembly of large messages.

- Flow control mechanisms.

- Timing parameters and error handling.

# Requirements

## RS_TP_01

Protocol Data Units (PDUs)

- SingleFrame (SF) - For unsegmented messages.

- FirstFrame (FF) - First segment of a multi-frame message.

- ConsecutiveFrame (CF) - Subsequent segments of a multi-frame message.

- FlowControl (FC) - Controls consecutive frame transmission rate.

## RS_TP_02

Segmentation and Reassembly

- If the data cannot be sent into a single frame, then the data is segmented

- One FirstFrame (FF) is sent containing the initial data and the total data length which is then followed by the consecutive frames which consist of SequenceNumbers (SN).

- These SequenceNumbers (SN) are used at the receiver to reassemble the segmented data.

## RS_TP_04

Protocol control information bit values

| Byte #1 Bits 7- 4 | Description |
| --- | --- |
| 0x00 | SingleFrame For unsegmented messages with CAN_DL ≤ 8, the message length is embedded in lower nibble of the only PCI byte (Byte #1). For unsegmented messages with CAN_DL > 8, the SingleFrame escape sequence shall be used where the lower nibble of the first PCI byte (Byte #1) is set to 00002 and the message length is embedded in the second PCI byte (Byte #2). SingleFrame (SF) shall be used to support the transmission of messages that can fit in a single CAN frame. |
| 0x01 | FirstFrame A FirstFrame (FF) shall only be used to support the transmission of messages that cannot fit in a single CAN frame, i.e. segmented messages. On receipt of a FirstFrame (FF), the receiving network layer entity shall start assembling the segmented message. — For segmented messages with a message length <= 4 095, the lower nibble of the first PCI byte (Byte #1) and the second PCI byte (Byte #2) includes the message length. — For segmented messages with a message length > 4 095, the FirstFrame escape sequence shall be used where the lower nibble of the first PCI byte (Byte #1) is set to 00002 and the second PCI byte (Byte #2) is set to zero. The message length is embedded in the following four PCI bytes (Byte #3 .. Byte #6, MSB first). |
| 0x02 | ConsecutiveFrame When sending segmented data, all consecutive frames following the FF are encoded as Con secutiveFrame (CF). On receipt of a ConsecutiveFrame (CF), the receiving network layer entity shall assemble the received data bytes until the whole message is received. The receiving enti ty shall pass the assembled message to the adjacent upper protocol layer after the last frame of the message has been received without error. |
| 0x03 | FlowControl The purpose of FlowControl (FC) is to regulate the rate at which CF N_PDUs are sent to the receiver. Three distinct types of FlowControl (FC) protocol control information are specified to support this function. The type is indicated by a field of the protocol control information called FlowStatus (FS). |
| 0x04 - 0x0F | Reserved This range of values is reserved by this part of ISO 15765. |

## RS_TP_03

Flow Control frame format

| Byte Position | Field Name | Description |
| --- | --- | --- |
| Byte 0 | PCI (Protocol Control Information) | First nibble indicates Flow Control PDU, second nibble indicates the FlowStatus |
| Byte 1 | BlockSize (BS) | Number of Consecutive Frames (CF) allowed before next FC frame. |
| Byte 2 | Separation Time Min(STmin) | Minimum delay between consecutive frames. |
| Byte 3-7 | Reserved | Unused bytes (set to 0x00) |

## RS_TP_05

Flow Status

| Value | Description |
| --- | --- |
| 0x00 | ContinueToSend (CTS) The FlowControl ContinueToSend parameter shall be encoded by setting the lower nibble of the N_PCI byte #1 to “0”. It shall cause the sender to resume the sending of ConsecutiveFrames. The meaning of this value is that the receiver is ready to receive a maximum of BS number of ConsecutiveFrames. |
| 0x01 | Wait (WAIT) The FlowControl Wait parameter shall be encoded by setting the lower nibble of the N_ PCI byte #1 to “1”. It shall cause the sender to continue to wait for a new FlowControl N_PDU and to restart its N_BS timer. If FlowStatus is set to Wait, the values of BS (BlockSize) and STmin (SeparationTime minimum) in the FlowControl message are not relevant and shall be ignored. |
| 0x02 | Overflow (OVFLW) The FlowControl Overflow parameter shall be encoded by setting the lower nibble of the N_ PCI byte #1 to “2”. It shall cause the sender to abort the transmission of a segmented mes sage and make an N_USData.confirm service call with the parameter = N_BUFF ER_OVFLW. This N_PCI FlowStatus parameter value is only allowed to be transmitted in the FlowControl N_PDU that follows the FirstFrame N_PDU and shall only be used if the message length FF_DL of the received FirstFrame N_PDU exceeds the buffer size of the receiving entity. If FlowStatus is set to Overflow, the values of BS (BlockSize) and STmin (SeparationTime mini mum) in the FlowControl message are not relevant and shall be ignored |
| 0x03 – 0x0F | Reserved This range of values is reserved by this part of ISO 15765 |

## RS_TP_06

Block Size

| Value | Description |
| --- | --- |
| 0x00 | BlockSize (BS) The BS parameter value 0 shall be used to indicate to the sender that no more FC frames shall be sent during the transmission of the segmented message. The sending network layer entity shall send all remaining ConsecutiveFrames without any stop for further FC frames from the receiving network layer entity. |
| 0x01 – 0xFF | BlockSize (BS) This range of BS parameter values shall be used to indicate to the sender the maximum number of ConsecutiveFrames that can be received without an intermediate FC frame from the receiving network entity. |

## RS_TP_07

SeparationTime minimum

| Value | Description |
| --- | --- |
| 0x00 – 0x7F | SeparationTime minimum (STmin) range: 0 ms – 127 ms The units of STmin in the range 0016 – 7F16 (0 – 127) are absolute milliseconds (ms). |
| 0x80 – 0xF0 | Reserved This range of values is reserved by this part of ISO 15765 |
| 0xF1 – 0xF9 | SeparationTime minimum (STmin) range: 100 µs – 900 µs The units of STmin in the range F116 – F916 are even multiples of 100 µs, where parameter value F116 represents 100 µs and parameter value F916 represents 900 µs. |
| 0xFA – 0xFF | Reserved This range of values is reserved by this part of ISO 15765 |

## RS_TP_08

SequenceNumber (SN)

- The SN shall start with zero for all segmented messages; the FirstFrame (FF) shall be assigned the value zero; it does not include an explicit SequenceNumber in the PCI field but shall be treated as the segment number zero.

- The SN of the first CF immediately following the FF shall be set to one.

- The SN shall be incremented by one for each new CF that is transmitted during a segmented message transmission.

- The SN value shall not be affected by any FlowControl (FC) frame.

- When the SN reaches the value of 15, it shall wraparound and be set to zero for the next CF.

| Byte #1 Bits 3 - 0 | Description |
| --- | --- |
| 0x00 – 0x0F | SequenceNumber (SN) The SequenceNumber (SN) shall be encoded in the lower nibble bits byte #1. The SN shall be set to a value within the range of 0 to 15. |

# References

SAE J1979 Revised APR2002