# OBD Modes Requirements Document

Contents

Document Control	3

Document History	4

Introduction	5

Functional Description	5

Requirements	5

References	8

# Document Control

- Document Title: OBD Stack service $04

- Version: 1.0

- Date: 31/3/25

- Author: Akshay Naik

- Reviewed by: [Enter Reviewer Name]

- Approved by: [Enter Approver Name]

# Document History

| Author | Date | Revision | Changes |
| --- | --- | --- | --- |

# Introduction

Service $04 allows for the clearing of stored DTCs and resetting of the Malfunction Indicator Light (MIL), commonly known as the "Check Engine" light. After repairing the underlying issue, this service helps reset the system. Clearing the DTCs also erases freeze frame data, which contains snapshots of engine conditions at the time of fault detection. However, if the issue persists, the ECU will store new trouble codes, ensuring continuous fault detection and system reliability.

# Functional Description

This document captures the requirements related to Service $04 – Clear Diagnostic Trouble Codes (DTCs) for the OBD stack. It defines the functional behaviour when a diagnostic tool sends a request to erase stored and pending DTCs, reset the Malfunction Indicator Light (MIL), and clear freeze frame data. The service allows the vehicle to restart its fault detection process after repairs.

# Requirements

## RS_OBD_01

P2 and P3 timings are as follows

| Parameter | Minimum value (ms) | Maximum value (ms) | Description |
| --- | --- | --- | --- |
| P2CAN | 0 | 50 | Time between external test equipment request message and the receipt of all unsegmented response messages and all first frames of segmented response message(s). Each OBD ECU shall start sending its response message within P2CAN after the request message has been correctly received. |
| P2*CAN | 0 | 5000 | Time between the successful reception of a negative response message with response code $78 and the next response message (positive or negative message). |

## RS_OBD_02

| Hex Value | Parameter name | Mnemonic |
| --- | --- | --- |
| 10 | generalReject This response code indicates that the service is rejected but the server (ECU) does not specify the reason of the rejection. | GR |
| 11 | serviceNotSupported This response code indicates that the requested action will not be taken because the server (ECU) does not support the requested service. | SNS |
| 12 | subFunctionNotSupported-InvalidFormat This response code indicates that the requested action will not be taken because the server (ECU) does not support the arguments of the request message or the format of the argument bytes do not match the prescribed format for the specified service. | SFNSIF |
| 21 | busy-RepeatRequest This response code indicates that the server (ECU) is temporarily too busy to perform the requested operation. For ISO 15765-4 protocol the client (external test equipment) shall behave as defined in ISO 15765-4. In a multi-client (more than one external test equipment, e.g., telematic client) environment the diagnostic request message of one client might be blocked temporarily by a negative response message with response code $21 while another client finishes a diagnostic task. Therefore this negative response code is only allowed to be used during the initialisation sequence of the protocol. NOTE If the server (ECU) is able to perform the diagnostic task but needs additional time to finish the task and prepares the response message, the negative response message with response code $78 shall be used instead of $21. | BRR |
| 22 | conditionsNotCorrectOrRequestSequenceError This response code indicates that the requested action will not be taken because the server (ECU) prerequisite conditions are not met. This request may also occur when sequence sensitive requests are issued in the wrong order. | CNCORSE |
| 78 | requestCorrectlyReceived This response code indicates that the request message was received correctly, and that any parameters in the request message were valid, but the action to be performed may not be completed yet. This response code can be used to indicate that the request message was properly received and does not need to be re-transmitted, but the server (ECU) is not yet ready to receive another request. The negative response message with this response code may be repeated by the ECU(s) within P2K-Line = P2CAN = P2*max until the positive response message with the requested data is available. | RCR-RP |

## RS_OBD_03

DTC request frame format

| Date byte | Parameter name | Cvt | Hex value | Mnemonic |
| --- | --- | --- | --- | --- |
| #1 | Clear/reset emission-related diagnostic information request SID | M | 04 | SIDRQ |

## RS_OBD_04

DTC response frame format

| Date byte | Parameter name | Cvt | Hex value | Mnemonic |
| --- | --- | --- | --- | --- |
| #1 | Clear/reset emission-related diagnostic information response SID | M | 44 | SIDPR |

# References

SAE J1979 Revised APR2002